# Supplementary material: From global heat-allocation bounds to a real waste-heat pathway

This supplement is generated from the canonical result files. It reports complete diagnostic summaries and does not introduce additional empirical data.

# Supplementary methods

The exact spatial search processes each six-hourly field in time order. Candidate sinks within the maximum great-circle distance are identified with a spherical cKDTree and rescored with exact haversine distance. Temporal candidates are vectorized over lag steps. Joint allocation compares spatial, temporal, and combined candidates under the same objective and eligibility masks.

For finite Q and quadratic burden, source benefit is evaluated analytically as P(2sΔ - Δ²) when s > Δ and Ps² when 0 < s ≤ Δ. Sink burden is P(2sΔ + Δ²) when s ≥ 0 and P max(s + Δ, 0)² otherwise. These stable expressions avoid subtracting nearly equal large burden values.

Validation checks scenario counts, objective identities, maximum distance, temporal same-location behavior, source and sink lookups against the processed cube, energy conservation, no-relocation retention, finite-Q monotonicity, seasonal group coverage, trajectory conservation, and explicit noncausal radiation interpretation.

# Supplementary tables

Table S1. Seasonal and local day-night marginal-burden summary.

| Season | Local Period | Eligible Cell Times | Marginal Burden Median | Marginal Burden P95 | Marginal Burden P99 | Marginal Burden Max | Total Human Burden |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DJF | day | 200391 | 1.18 | 24.1 | 71.9 | 513 | 4.5e+13 |
| DJF | night | 138962 | 0.562 | 16.5 | 58.1 | 299 | 2.33e+13 |
| JJA | day | 354292 | 1.62 | 63.5 | 178 | 778 | 1.8e+14 |
| JJA | night | 234431 | 1.47 | 63.7 | 179 | 649 | 1.12e+14 |
| MAM | day | 242152 | 1.48 | 45.9 | 136 | 707 | 9.44e+13 |
| MAM | night | 159273 | 1.06 | 42.1 | 130 | 527 | 5.49e+13 |
| SON | day | 258413 | 1.7 | 44.3 | 129 | 710 | 9.71e+13 |
| SON | night | 179217 | 1.24 | 42.3 | 121 | 540 | 6.16e+13 |

Table S2. Source-hotspot robustness summary.

| Metric | Source Country | Scenarios | Minimum Source Marginal | Maximum Source Marginal |
| --- | --- | --- | --- | --- |
| air_temperature | Bangladesh | 3 | 448 | 561 |
| humidex | Bangladesh | 27 | 7.06 | 8.43e+04 |
| wet_bulb_proxy | Bangladesh | 3 | 34.2 | 65.8 |

Table S3. Progressive exclusion-mask ablation results.

| Scenario | Source Time | Source Latitude | Source Longitude | Sink Latitude | Sink Longitude | Sink Is Ocean | Sink Is Cryosphere | Distance Km | Source Marginal Benefit | Sink Marginal Burden | Net Benefit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| unrestricted_global | 2023-06-05T00:00:00 | 23.8 | 90 | -35.2 | 219 | True | False | 1.5e+04 | 778 | 0 | 778 |
| distance_only | 2023-06-05T00:00:00 | 23.8 | 90 | -12.4 | 116 | True | False | 4.93e+03 | 778 | 0 | 778 |
| plus_cryosphere_exclusion | 2023-06-05T00:00:00 | 23.8 | 90 | -12.4 | 116 | True | False | 4.93e+03 | 778 | 0 | 778 |
| plus_ocean_exclusion | 2023-06-05T00:00:00 | 23.8 | 90 | 44.8 | 135 | False | False | 4.67e+03 | 778 | 0 | 778 |
| plus_ocean_and_cryosphere_exclusion | 2023-06-05T00:00:00 | 23.8 | 90 | 44.8 | 135 | False | False | 4.67e+03 | 778 | 0 | 778 |

Table S4. Finite-transfer and local-capacity sensitivity.

| Energy Gj | Status | Source Time | Source Latitude | Source Longitude | Sink Latitude | Sink Longitude | Distance Km | Source Temperature Change K | Sink Temperature Change K | Source Benefit Total | Sink Burden Total | Transport Penalty Total | Net Benefit Total | Net Benefit Per Gj |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | feasible | 2023-06-05T00:00:00 | 23.8 | 90 | 27.6 | 90 | 424 | 2.01e-07 | 2.08e-07 | 778 | 0 | 0.00424 | 778 | 778 |
| 1e+03 | feasible | 2023-06-05T00:00:00 | 23.8 | 90 | 27.6 | 90 | 424 | 0.000201 | 0.000208 | 7.78e+05 | 0 | 4.24 | 7.78e+05 | 778 |
| 1e+05 | feasible | 2023-06-05T00:00:00 | 23.8 | 90 | 27.6 | 90 | 424 | 0.0201 | 0.0208 | 7.77e+07 | 0 | 424 | 7.77e+07 | 777 |
| 1e+06 | infeasible_under_local_capacity | nan | nan | nan | nan | nan | nan | nan | nan | nan | nan | nan | nan | nan |

Table S5. Reduced-order natural-redistribution diagnostic.

| Horizon Hours | Latitude | Longitude | Distance From Sink Km | Distance From Source Km | Gaussian Dispersion Radius Km | Nearest Cell Population | Nearest Cell Cryosphere | Energy Retained Fraction | Model Scope |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 30.2 | 93.2 | 419 | 774 | 13.1 | 1.63e+05 | True | 1 | 10m-wind Lagrangian trajectory with assumed Gaussian dispersion; not a climate or heat-transport model |
| 72 | 34.7 | 90.4 | 785 | 1.21e+03 | 22.8 | 0 | True | 1 | 10m-wind Lagrangian trajectory with assumed Gaussian dispersion; not a climate or heat-transport model |
| 168 | 31.2 | 90.4 | 399 | 822 | 34.8 | 5.27e+04 | False | 1 | 10m-wind Lagrangian trajectory with assumed Gaussian dispersion; not a climate or heat-transport model |

Table S6. Prespecified structural-replication results.

| Scenario Id | Climate Product | Climate Year | Population Product | Spatial Coarsening Factor | Thermal Metric | Source Country | Spatial Zero Penalty Sink Country | Spatial Zero Penalty Net Benefit Per Gj | Temporal To Spatial Value Ratio | Source Shift Km From Canonical | Zero Penalty Sink Shift Km From Canonical |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ncep1_2010_worldpop_native_humidex | NCEP/NCAR Reanalysis 1 | 2010 | WorldPop unconstrained | 1 | humidex | Bangladesh | Russia | 787 | 0.506 | 0 | 0 |
| ncep1_2015_worldpop_native_humidex | NCEP/NCAR Reanalysis 1 | 2015 | WorldPop unconstrained | 1 | humidex | Bangladesh | Russia | 763 | 0.556 | 0 | 0 |
| ncep1_2020_worldpop_native_humidex | NCEP/NCAR Reanalysis 1 | 2020 | WorldPop unconstrained | 1 | humidex | Bangladesh | Russia | 766 | 0.404 | 0 | 0 |
| ncep1_2023_worldpop_native_humidex | NCEP/NCAR Reanalysis 1 | 2023 | WorldPop unconstrained | 1 | humidex | Bangladesh | Russia | 778 | 0.539 | 0 | 0 |
| ncep1_2023_ghsl_native_humidex | NCEP/NCAR Reanalysis 1 | 2023 | GHS-POP R2023A | 1 | humidex | Bangladesh | Russia | 787 | 0.539 | 0 | 0 |
| ncep1_2023_worldpop_coarse_humidex | NCEP/NCAR Reanalysis 1 | 2023 | WorldPop unconstrained | 2 | humidex | India | Russia | 325 | 0.428 | 306 | 345 |
| ncep1_2023_worldpop_native_air_temperature | NCEP/NCAR Reanalysis 1 | 2023 | WorldPop unconstrained | 1 | air_temperature | Bangladesh | Russia | 504 | 1 | 0 | 1.61e+03 |
| ncep1_2023_worldpop_native_wet_bulb | NCEP/NCAR Reanalysis 1 | 2023 | WorldPop unconstrained | 1 | wet_bulb_proxy | Bangladesh | Russia | 50 | 0.602 | 0 | 0 |
| ncep2_2023_worldpop_native_humidex | NCEP-DOE Reanalysis 2 | 2023 | WorldPop unconstrained | 1 | humidex | Bangladesh | Russia | 649 | 0.394 | 0 | 0 |

# Supplementary figure legends

Figure S1. Source-hotspot selections in the limited one-year parameter sweep. Humidex varies threshold, curvature, and prescribed mixing height; alternative metrics vary threshold.

Figure S2. Net benefit per GJ across finite transferred-energy levels; the dashed line identifies infeasibility under the local temperature cap.

Figure S3. Seven-day reduced-order 10-m-wind trajectory from the selected release cell, with 24-, 72-, and 168-hour positions.

Figure S4. Annual peak population-weighted marginal thermal-stress burden per GJ on the common global grid.

Figure S5. Seasonal day-night variation in the 95th percentile of marginal thermal-stress burden.

Figure S6. Progressive mask ablation showing how distance, cryosphere, and ocean exclusions change the representative sink among non-unique candidates.

Figure S7. Selected allocation mode and loss from the zero-penalty optimum across abstract transport and storage penalty coefficients.